### 1. Requirements

- Devices Needed:

  - Cisco Router 2811

  - Cisco Switch 2960

  - 1 Laptop

  - 1 PC

- Software:

  - Cisco Packet Tracer

- Objective:

  - Connect Router, Switch, Laptop, and PC.

  - Configure basic IP addressing.

  - Configure DHCP

### 2. Network Topology

- Router 2811 connected to Switch 2960 Gigabit 0/0.

- Laptop connected to Switch 2960 via FastEthernet port.

- PC connected to Switch 2960 via FastEthernet port.

_________________

### 3. Configuration Steps

A. Router Configuration

Router> enable

Router# configure terminal

Router(config)# hostname R1

R-1 (config)# interface gig0/0

R-1 (config-if)# ip address 192.168.10.1 255.255.255.0

R-1 (config-if)# no shutdown

R-1 (config-if)# exit

R-1 (config)# banner motd !Welcome To Net_Tech #Warning! Authorized users only! DHCP_Configuration!

R-1 (config)# exit

R-1# write

R-1(config)# username DHCP password DHCP

R-1(config)# line Console

R-1(config-line)# login local

R-1(config-line)#exit

R-1(config)# do write

### 4. DHCP Configuration on Router

Step 1: Enable DHCP and define pool

R1> enable

R1# configure terminal

R1(config)# ip dhcp pool LAN-Network

R1(dhcp-config)# network 192.168.10.0 255.255.255.0

R1(dhcp-config)# default-router 192.168.10.1

R1(dhcp-config)# dns-server 8.8.8.8

R1(dhcp-config)# exit

Exclude Router IP from DHCP

R1(config)# ip dhcp excluded-address 192.168.10.1 192.168.10.10

### 6. Laptop and PC Configuration for DHCP

- Set IP configuration to Obtain IP Address Automatically (DHCP).

- After connecting to switch, the devices will get:

  - Laptop: e.g., 192.168.10.11

  - PC: e.g., 192.168.10.12

  - Gateway: 192.168.10.1

  - DNS: 8.8.8.8